Good Manufacturing Practices

STATEMENT

Good personal hygiene policies and practices are the foundation for successful food safety and quality assurance in all food manufacturing facilities. Personnel have the most magnitude and direction of microorganisms, chemical residues and bringing of foreign materials into the food facility, and as such, can be a source of unwanted contamination to products.

OBJECTIVE

Below is a simple list of Do’s and Don't’s when it comes to a High-Risk Barrier.

THINK of it like a balloon and you could be the pin. One poor action could be enough to breach the barrier and burst the balloon.

- DO wear the correct colored hairnets – Blue for Low-risk workers and red distinguish High-risk food handlers, YELLOW skilled workers, White for Management, Green for qualified first aiders. This makes the policy easier to implement and enforce and new staff, visitors to understand whom they may need to seek advice from.

- DON’T pick up any items from the floors. Whether it is in a packet or in a raw state. Corrective actions should be reporting accidental spillages to the line cleaner, line manager or hygiene team.

- DO wash your hands immediately if you come into contact with any item from the floor. Change all PPE/PPC dispose of disposable PPE in the waste bins provided.

- DO ensure you are wearing a barrier between yourself and the product. Items include; coats/overalls, plastic aprons or plastic sleeves, hairnets and balaclavas, and gloves, when appropriate.

- DON’T wear protective clothing outside the production areas; these must be changed regularly when visually dirty and stored in the hooks provided in the entrance areas.

- DON’T wear jewelry in the production areas, period! Tongue rings, earrings and any other body piercings, including those that are covered are not to be worn in a production area. This prevents foreign material from contaminating products and reduces micro-bacterial growth. One plain ring may be worn only.

- DO regularly checks on footwear and clothing for frayed edges or loose snaps, report the damaged item to the laundry store by removing the item from the area and select a new clean coat from the entrance storage rack.

- DO ensure that all external footwear is removed prior to swinging legs into the internal boot area. This has to be an effective barrier against microbial contamination that travels from footwear through food-contact areas low risk to high-risk areas.

- DON’T remove internal footwear from the boot areas, all boots must remain at the entrance of the departments stored on the racking provided.

- DO ensure that all personnel wash their hands and follow the departmental entrance procedures. All personnel is subjected to washing their hands prior to entry. Personnel must acknowledge the signage at the entrance and follow the step by step guides displayed. This policy also includes Visitors, Contractors etc. All staff must be aware of the company’s Handwashing policy they are required to wash their hands after any type of activity that could contaminate the hands with pathogens, including using the toilets, blowing the nose or touching any body parts, handling of raw food, waste or non-food-contact surfaces such as light switches or pipes, and door handles etc. Random use of hand swabs is used to verify hand cleanliness. Turnstiles to the high-risk areas are used to sanitize the hands where handwashing may have been missed. All personnel must rub the hands effectively for reduction of bacterial growth.

- DO ensure that any packaging from a low-risk area transferred to a high-risk area is thoroughly wiped and sanitized; using sanitizing wipes prior to accepting. There must be a minimum contact time of 15 minutes prior to the removal of barrier bags.

- DON’T allow tools and utensils to become a source of contamination. From a GMP standpoint, we have to make sure that hand tools and utensils such as knives and scissors that are used by personnel in food production activities do not become a source of product contamination. (i.e., the employee uses scissors to open a plastic bag of allergen-containing ingredient and then uses the same scissors to open a container of non-allergen-containing ingredient). All tools must be stored in the area and no external utensils and tools brought into the area. This includes Pens and other items used in the area. No items to be removed from personal pockets such as keys, pens or any other item, as these become a source of contamination.

- DO ensure that all doors are kept closed at all times. This includes loading bays, Fire Exits, Entrance doors etc. The area is designed to have high-pressure air flow to prevent drafts from low-risk areas and dust entering the area. This change in pressure can be felt at the entrance when entering.

- DON’T reach into a low-risk environment to i.e. shake hands with a colleague, Pass items to one another without sanitizing the item. It must be clearly understood that there is a difference between a low-risk operative and a high-risk operative. Only high-risk operative have clearance to work in a high-risk environment. Stool testing and other means such as medical screening declarations are used to determine the placement of employees.

- DO ensure that when returning from exotic or foreign holidays that clearance is given by occupational health before entering the high-risk areas.

- DO ensure you are medically fit and are not suffering from food poisoning symptoms. If anyone has suffered from a bout of food poisoning, this must be declared to occupational health that will provide clearance before entry into the high-risk environment.

- DO control anything that moves within the plant this, includes tool carts, scales, ladders, pallet trucks, forklifts, cleaning chemicals, and sanitizer containers, and hoses etc.

- DO ensure that you have the appropriate resources to do your job. The ideal situation is that all tools are company-owned, are all turned in at the end of every production shift, we do not want employees bringing tools from home and raise the possibility of outside contaminants entering the plant. Integrity checks and booking in/out must be maintained for the condition of the items.

- DO make sure that production tools are cleanable and maintained in good condition. For example, scratches or cracks in the handle of a food-contact knife create a great niche for microbial growth and contamination.

- DO control the number of tools and supplies that are taken out to production areas. The company approved pens are only allowed in the production areas. These can be obtained from the management.

- DO not remove maintenance tools from the production areas. Engineers are highly mobile within the plant, moving from clean to dirty and/or raw to ready-to-eat areas of the plant as maintenance tasks dictate. Therefore, segregation of maintenance personnel and their tools and equipment is recommended to prevent contamination. Our, mechanics to the department, their tools, and equipment dedicated to the department, and they have a cleaner and sanitizer available to them for use prior to working on any food-contact surface

- DO designate or dedicate sanitation personnel, tools, and equipment. Color-coding of sanitation tools is a tremendous tool because it works and makes these tools easily distinguishable from other production tools. Familiarise yourself with the color coding for the tools you require in your task.

- DON’T allow food-contact surface equipment to be placed on the floor during sanitation disassembly/ reassembly operations.

- DO follow the company’s policies and procedure. If you are unsure training is available and essential to educate employees and reinforce good personal hygiene practices. If you are unsure of anything then please let us know. Training Instructors/Line Managers etc. are there to assist you.

- DO incorporate personal hygiene practices as a topic of discussion on a daily basis or during break times. This will help to manage your areas and help to provide you with a pleasant environment to work in.

- DO employ the visual aids to create continual employee awareness of personal hygiene best practices i.e. showing the sequential steps of hand washing procedures. Use the visual aids to make your tasks easier and understand the requirements of the customer you are supplying. Providing safe, hygienic, good quality ready to eat foods. Take pride in the products you handle, if it's not to your standard then it’s not the standard of the customers. Report poor quality products to the line manager.

SUMMARY

Ultimately, all employees want/need to be part of the food safety team; they must do the right things. If we do the job properly in terms of personal hygiene practices, sanitation, and the tasks we do. Potential sources of contamination will be significantly reduced.

REMEMBER – we produce foods for others to eat and we are bound by law to protect the consumer and not put them to risk.